BURGER

BURGER

BlackPill INTERFACE

for STM32F4xx BlackPill Development Board

Datasheet

Authors:	Gerhard Burger

Last update:	22.05.2025

File:	input.doc

Attachments:	no

Language:	English

## Versions

| Version | date | Remarks |
| --- | --- | --- |
| V1.0 | 22.05.2025 | first version |

Table of content

Versions	1

1	Introduction	3

1.1	Common	3

1.2	Scope of this datasheet	3

2	Typical applications	3

3	Power supply	3

4	IO-Features	3

4.1	Features of diagnosis adapter board (top)	4

4.2	Features of diagnosis adapter board (bot)	4

5	Technical data	4

5.1	Environment limits	4

5.2	Dimensions	5

5.3	Weight	5

6	Disclaimer	5

# Introduction

## Common

The STM32F4xx BlackPill interface is designed to expand the capabilities of the BlackPill STM32F411 development board. It provides robust I/O functionality, reliable power separation, and versatile communication options, making it ideal for industrial automation, motor control, and embedded system prototyping.

## Scope of this datasheet

This datasheet describes the features and functionality of the STM32F4xx BlackPill interface.

# Typical applications

- learning

- Standalone control systems

- Encoder interface applications

- Remote sensor recognition e.g. interfacing HC05 Bluetooth module

# Power supply

- Input Voltage: 24 VDC

- Separate power domains for logic and PROFET outputs for improved safety and noise isolation

# Features

- 3x Encoder Interfaces

  - A, B, and Index inputs

  - Selectable encoder supply voltage: 5V or 24V

- 3x PROFET High-Side Switch Outputs (Optional)

  - 24V output control for actuators or indicators

- 1x Analog Output

  - Voltage range: 0–10V for control signals

- External Communication Module Interface

  - 4 configurable I/O pins supporting RS232, SPI, I²C, or CAN

  - Suitable for external modules such as Bluetooth, Wi-Fi, or CAN transceivers

- 8 x General Purpose I/O

  - Configurable as SPI, RS232, digital input/output, analog input, or encoder home signal

- 2x General Purpose Status LEDs

- 1x Buzzer

- I/O lines feature protection circuitry for enhanced durability

- Fits to hat rail case or fully closed plastic case

## Features of diagnosis adapter board (top)

6

Contact points for

voltage measurement

1 mm stainless steel case

319 mm x 244 mm x 36 mm

Stainless steel cover

Second terminal for current measurement

Figure 1: Features of STM32F4xx BlackPill interface

## Features of diagnosis adapter board (bot)

6

Contact points for

voltage measurement

1 mm stainless steel case

319 mm x 244 mm x 36 mm

Stainless steel cover

Second terminal for current measurement

Figure 2: Features of STM32F4xx BlackPill interface

# Case

## Hat rail case

## IP67 plastic case

# Technical data

## Environment limits

Development purpose

## Dimensions

Figure 3: Dimension parameters Diagnosis Adapter

| Pos | Parameter | Value |
| --- | --- | --- |
| 1 | Length [l] | 76.25 mm |
| 2 | Width [w] | 72 mm |
| 3 | Height [h] | 25 mm |

Figure 4: Dimensions or BlackPill interface board

## Weight

| Pos | Description | Value |
| --- | --- | --- |
| 1 | STM32F4xx BlackPill interface | 80 g |

Figure 5: Weight of BlackPill interface board

# Disclaimer

STM32F is a registered trademark of DuPont Corporation.

CONTACT BURGER Montafonerstraße 17 6771 St. Anton i. M. AUSTRIA geburger@iplace.at +43-(0)650 44 15 94 www.mcda.at