TN001

# Quick and dirty steps to run a CNC-program with AKKON

Version 1.0

Test AKKON in 5 minutes

Authors:	Gerhard Burger

Version:	1.0

Last update:	22.06.2005

File:	Quick and dirty steps to run a CNC_V1_4.doc

Attachments:	-no attachments

## Table of versions

| Version | Date | Remarks |
| --- | --- | --- |
| 1.0 | 11.05.2005 | First version. |
| 1.1 | 21.05.2005 | Some small changes in names |
| 1.2 | 14.06.2005 | Some small updates improving the installation |
| 1.3 | 15.06.2005 | AKKON CNC Controller can now be called by a batch file, setting up the right COM-port |
| 1.4 | 21.06.20051 | Some small changes regarding startup of AKKON |

# Table of content

Quick and dirty steps to run a CNC-program with AKKON	1

Table of versions	1

Table of content	2

1	Introduction	2

1.1	File Installation	3

1.2	Step 0: Start AKKON CNC Controller on the DOS machine	4

1.3	Step1: Start AKKON Desk	5

1.4	Step 2: Connect to AKKON CNC controller by pressing the green (red button	6

1.5	Step 3: Move to part zero point (starting point of CNC program)	6

1.6	Step 4: Set feed rate with the feed scroll bar to maximum	7

1.7	Step 5: Select File open CNC file and select the file “Schild.txt”	7

1.8	Step 6: Select the Sim button and press it so that the simulation mode is on.	8

1.9	Step 7: Press the start button and run	8

2	Disclaimer:	10

2.1	Limited Warranty and Disclaimer of Warranty	10

2.2	ACKNOWLEDGMENT	10

# Introduction

This document outlines how to run a CNC-program with AKKON without setting up machine parameters. The document relates to AKKON Desk and AKKON CNC-Controller. If you do not have the necessary hardware (Personal computer with MS-DOS and a serial null modem-cable) AKKON can be tested using the AKKON Simulator. Information about the AKKON Simulator can be found in Technical Note TN002

AKKON

CNC Controller

(PC with DOS

or micro

controller)

RS232

AKKON control

cable

Centronics

RS232

Centronics

RS232

machine controller hardware

AKKON Desk

(PC with MS Windows 2000, XP)

Figure 1: Concept of AKKON

Note: Do the quick and dirty test run without a CNC machine. It is just for demonstration use.

## File Installation

- Copy file AKKON.Zip to your Windows-PC

- Unzip AKKON.Zip

- Install LCD-Fonts (LCDGeri.TTF and LCDn.TTF) from the directory /fonts in your MS-Windows system.

Figure 2: Installation of LCD-Font

- Copy files from sub directory  AKKON/AKKON_CNC_CONTROLLER/DOS to the DOS PC

- Connect the PC running AKKON.exe over serial null modem-cable with AKKON CNC Controller

Figure 3: Connection between AKKON Desk and AKKON CNC Controller

If no null mode-cable is available, the cable can easily be built according to the picture below.

Figure 4: Serial Null Modem Cable

## Step 0: Start AKKON CNC Controller on the DOS machine

From here, it is assumed, that the AKKON CNC controller is already connected with AKKON Desk over a RS232 Null-Modem cable. (see also the AKKON web page for cable configuration).

The AKKON CNC controller is started by calling the DOS program AKKON_DOS.EXE. Please run the attached batch file for startup of the AKKON CNC Controller program settings up the right communication port.

Running AKKON_DOS with COM1: call AkkonCom1.bat. If you want to use another COM-Port, please call the related batch file (AkkonCom2.bat, AkkonCom3.bat or AkkonCom4.bat).

## Step1: Start AKKON Desk

Figure 5: AKKON Desk after program start

## Step 2: Connect to AKKON CNC controller by pressing the green (red button

Figure 6: AKKON Desk after establishing a connection to the AKKON CNC controller

## Step 3: Move to part zero point (starting point of CNC program)

Set feed rate and press Cursor arrows and F1/F2 for up and down to move the spindle head to zero point of the part.

Press button Wx and OK

Press button Wy and OK

Press button Wz and OK

Figure 7: Tool buttons for setting the zero point of the part

## Step 4: Set feed rate with the feed scroll bar to maximum

Figure 8: Scrollbar for controlling the feed and the speed

## Step 5: Select File open CNC file and select the file “Schild.txt”

Pleas set the feed scrollbar to maximum.

Figure 9: File open dialog for loading a CNC file

Now the program is uploading the instructions to the AKKON controller.

## Step 6: Select the Sim button and press it so that the simulation mode is on.

| Button | Description |
| --- | --- |
|  | Simulation is off |
|  | Simulation is on |

Figure 10: Toolbar buttons for switching on/off simulation mode

## Step 7: Press the start button and run

| Button | Description |
| --- | --- |
|  | Start Button |
|  | Pause program, press start to proceed |
|  | Stop and abort the program |

Figure 11: Toolbar buttons for controlling automatic mode

Figure 12: Simulation result after running a CNC program

The AKKON CNC-Controller can each time be stopped by pressing ESC. The program will then end.

Note: It is not necessary to wait until the program loading progress bar has indicated that the job has been done. The start button can be pressed at the same time as a new job will be uploaded.

# Disclaimer:

## Limited Warranty and Disclaimer of Warranty

THIS SOFTWARE AND ACCOMPANYING WRITTEN MATERIALS (INCLUDING INSTRUCTIONS FOR USE) ARE PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND. FURTHER, the author DOES NOT WARRANT, GUARANTEE, OR MAKE ANY REPRESENTATIONS REGARDING THE USE, OR THE RESULTS OF USE, OF THE SOFTWARE OR WRITTEN MATERIALS IN TERMS OF CORRECTNESS, ACCURACY, RELIABILITY, CURRENTNESS, OR OTHERWISE. THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THE SOFTWARE IS ASSUMED BY YOU. IF THE SOFTWARE OR WRITTEN MATERIALS ARE DEFECTIVE YOU, AND NOT the author OR ITS DEALERS, DISTRIBUTORS, AGENTS, OR EMPLOYEES, ASSUME THE ENTIRE COST OF ALL NECESSARY SERVICING, REPAIR, OR CORRECTION.

THE ABOVE IS THE ONLY WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, THAT IS MADE BY the author, ON THIS PRODUCT. NO ORAL OR WRITTEN INFORMATION OR ADVICE GIVEN BY the author, ITS DEALERS, DISTRIBUTORS, AGENTS OR EMPLOYEES SHALL CREATE A WARRANTY OR IN ANY WAY INCREASE THE SCOPE OF THIS WARRANTY AND YOU MAY NOT RELY ON ANY SUCH INFORMATION OR ADVICE.

NEITHER the author NOR ANYONE ELSE WHO HAS BEEN INVOLVED IN THE CREATION, PRODUCTION OR DELIVERY OF THIS PRODUCT SHALL BE LIABLE FOR ANY DIRECT, INDIRECT, CONSEQUENTIAL OR INCIDENTAL DAMAGES (INCLUDING DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF BUSINESS INFORMATION, AND THE LIKE) ARISING OUT OF THE USE OR INABILITY TO USE SUCH PRODUCT EVEN IF the author HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

## ACKNOWLEDGMENT

BY USING THIS PRODUCT YOU ACKNOWLEDGE THAT YOU HAVE READ THIS LIMITED WARRANTY, UNDERSTAND IT, AND AGREE TO BE BOUND BY ITS' TERMS AND CONDITIONS. YOU ALSO AGREE THAT THE LIMITED WARRANTY IS THE COMPLETE AND EXCLUSIVE STATEMENT OF AGREEMENT BETWEEN THE PARTIES AND SUPERSEDE ALL PROPOSALS OR PRIOR AGREEMENTS, ORAL OR WRITTEN, AND ANY OTHER COMMUNICATIONS BETWEEN THE PARTIES RELATING TO THE SUBJECT MATTER OF THE LIMITED WARRANTY.