TN002

# Running AKKON Simulator on Windows XP

Version 1.0

Authors:	Gerhard Burger

Version:	1.0

Last update:	22.06.2005

File:

Attachments:	-no attachments

Downloads:	AKKON.Zip

## Table of versions

| Version | Date | Remarks |
| --- | --- | --- |
| 1.0 | 21.05.2005 | AKKON simulator for MS Windows™ is releasd |

# Table of content

Running AKKON Simulator on Windows XP	1

Table of versions	1

Table of content	2

1	Introduction	3

2	Requirements	3

3	Concept of AKKON Simulator (Windows-Version)	3

3.1	Possibility 1: Running AKKON Desk and AKKON Simulator (Win) with a serial Null-Modem cable	3

3.2	Possibility 2: Running AKKON Desk and AKKON Simulator (Win) without a serial Null-Modem cable	4

4	Installation	4

5	Steps for running the AKKON software	5

5.1	Step 0: Start AKKON Simulator	5

5.2	Step1: Start AKKON Desk	6

5.3	Step 2: Connect to AKKON CNC controller by pressing the green (red button	7

5.4	Step 3: Connect to AKKON CNC controller by pressing the green (red button	8

5.5	Step 4: Move to part zero point (starting point of CNC program)	8

5.6	Step 5: Set feed rate with the feed scroll bar to maximum	9

5.7	Step 6: Select File open CNC file and select the file “Schild.txt”	9

5.8	Step 7: Select the Sim button and press it so that the simulation mode is on.	10

5.9	Step 8: Press the start button and run	10

6	Disclaimer:	12

6.1	Limited Warranty and Disclaimer of Warranty	12

6.2	ACKNOWLEDGMENT	12

# Introduction

This document outlines how to run a CNC-program with AKKON Desk and AKKON Simulator on the same personal computer.

# Requirements

- Personal Computer with MS Windows 2000 or MS Windows XP

- AKKON Desk

- AKKON Simulator for Windows XP

# Concept of AKKON Simulator (Windows-Version)

There are two possibilities running AKKON Desk with the AKKON Simulator for Windows.

## Possibility 1: Running AKKON Desk and AKKON Simulator (Win) with a serial Null-Modem cable

Figure 1: Running AKKON Desk and AKKON Simulator with a serial Null-Modem cable

In this case, the personal needs two free serial ports. These ports are connected together by a serial Null-Modem able. If no Null-Modem cable is available, the cable can easily be built according to the picture below.

Figure 2: Serial Null-Modem cable

## Possibility 2: Running AKKON Desk and AKKON Simulator (Win) without a serial Null-Modem cable

Figure 3: Running AKKON Desk and AKKON Simulator without a serial Null Modem cable

In this case, serial ports are created by a specifc software that creates “virtual serial ports”, available on the internet e.g. http://www.virtual-serial-port.com free 15days trial version. Just install the software and create two serial ports which are connected to each other.

# Installation

- Copy file AKKON.Zip to your Windows-PC

- Unzip AKKON.Zip

- Install LCD-Fonts (LCDGeri.TTF and LCDn.TTF) from the directory /fonts in your MS-Windows system.

Figure 4: Installation of LCD-Font

# Steps for running the AKKON software

## Step 0: Start AKKON Simulator

It is assumed, that the AKKON CNC controller is already connected with AKKON Desk over a RS232 NULL-MODEM CABLE (or over two virtual serial ports).

The AKKON Simulator is started by calling the Windows program AKKON_CNC_CONTROL_XP.exe. Please move to sub directory AKKON/AKKON_CNC_CONTROLLER/WIN_XP and run the attached batch file for startup of the Controller program settings up the right communication port.

Figure 5: Screen shot of AKKON Simulator running on MS Windows™

Running AKKON_CNC_CONTROL_XP.exe with COM1: call AKKON_CNC_CONTROL_XP_COM1.bat. If you want to use another COM-Port, please call the related batch file (AKKON_CNC_CONTROL_XP_COM1.bat etc.).

## Step1: Start AKKON Desk

Figure 6: AKKON Desk after program start

## Step 2: Connect to AKKON CNC controller by pressing the green (red button

Move to sub directory AKKON_Desk and start AKKON. Set the right com settings to establish a connection to the AKKON simulator.

Figure 7: Screen shot of AKKON Desk with Com settings dialog

## Step 3: Connect to AKKON CNC controller by pressing the green (red button

Figure 8: AKKON Desk after establishing a connection to the AKKON CNC controller

## Step 4: Move to part zero point (starting point of CNC program)

Set feed rate and press Cursor arrows and F1/F2 for up and down to move the spindle head to zero point of the part.

Press button Wx and OK

Press button Wy and OK

Press button Wz and OK

Figure 9: Tool buttons for setting the zero point of the part

## Step 5: Set feed rate with the feed scroll bar to maximum

Figure 10: Scrollbar for controlling the feed and the speed

## Step 6: Select File open CNC file and select the file “Schild.txt”

Pleas set the feed scrollbar to maximum.

Figure 11: File open dialog for loading a CNC file

Now the program is uploading the instructions to the AKKON controller.

## Step 7: Select the Sim button and press it so that the simulation mode is on.

| Button | Description |
| --- | --- |
|  | Simulation is off |
|  | Simulation is on |

Figure 12: Toolbar buttons for switching on/off simulation mode

## Step 8: Press the start button and run

| Button | Description |
| --- | --- |
|  | Start Button |
|  | Pause program, press start to proceed |
|  | Stop and abort the program |

Figure 13: Toolbar buttons for controlling automatic mode

Figure 14: Simulation result after running a CNC program

The AKKON Simulator can each time be stopped by pressing ESC. The program will then end.

Note: It is not necessary to wait until the program loading progress bar has indicated that the job has been done. The start button can be pressed at the same time as a new job will be uploaded.

# Disclaimer:

## Limited Warranty and Disclaimer of Warranty

THIS SOFTWARE AND ACCOMPANYING WRITTEN MATERIALS (INCLUDING INSTRUCTIONS FOR USE) ARE PROVIDED "AS IS" WITHOUT WARRANTY OF ANY KIND. FURTHER, the author DOES NOT WARRANT, GUARANTEE, OR MAKE ANY REPRESENTATIONS REGARDING THE USE, OR THE RESULTS OF USE, OF THE SOFTWARE OR WRITTEN MATERIALS IN TERMS OF CORRECTNESS, ACCURACY, RELIABILITY, CURRENTNESS, OR OTHERWISE. THE ENTIRE RISK AS TO THE RESULTS AND PERFORMANCE OF THE SOFTWARE IS ASSUMED BY YOU. IF THE SOFTWARE OR WRITTEN MATERIALS ARE DEFECTIVE YOU, AND NOT the author OR ITS DEALERS, DISTRIBUTORS, AGENTS, OR EMPLOYEES, ASSUME THE ENTIRE COST OF ALL NECESSARY SERVICING, REPAIR, OR CORRECTION.

THE ABOVE IS THE ONLY WARRANTY OF ANY KIND, EITHER EXPRESS OR IMPLIED, INCLUDING BUT NOT LIMITED TO THE IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR PURPOSE, THAT IS MADE BY the author, ON THIS PRODUCT. NO ORAL OR WRITTEN INFORMATION OR ADVICE GIVEN BY the author, ITS DEALERS, DISTRIBUTORS, AGENTS OR EMPLOYEES SHALL CREATE A WARRANTY OR IN ANY WAY INCREASE THE SCOPE OF THIS WARRANTY AND YOU MAY NOT RELY ON ANY SUCH INFORMATION OR ADVICE.

NEITHER the author NOR ANYONE ELSE WHO HAS BEEN INVOLVED IN THE CREATION, PRODUCTION OR DELIVERY OF THIS PRODUCT SHALL BE LIABLE FOR ANY DIRECT, INDIRECT, CONSEQUENTIAL OR INCIDENTAL DAMAGES (INCLUDING DAMAGES FOR LOSS OF BUSINESS PROFITS, BUSINESS INTERRUPTION, LOSS OF BUSINESS INFORMATION, AND THE LIKE) ARISING OUT OF THE USE OR INABILITY TO USE SUCH PRODUCT EVEN IF the author HAS BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES.

## ACKNOWLEDGMENT

BY USING THIS PRODUCT YOU ACKNOWLEDGE THAT YOU HAVE READ THIS LIMITED WARRANTY, UNDERSTAND IT, AND AGREE TO BE BOUND BY ITS' TERMS AND CONDITIONS. YOU ALSO AGREE THAT THE LIMITED WARRANTY IS THE COMPLETE AND EXCLUSIVE STATEMENT OF AGREEMENT BETWEEN THE PARTIES AND SUPERSEDE ALL PROPOSALS OR PRIOR AGREEMENTS, ORAL OR WRITTEN, AND ANY OTHER COMMUNICATIONS BETWEEN THE PARTIES RELATING TO THE SUBJECT MATTER OF THE LIMITED WARRANTY.